RelayIQ Adaptive Relay Intelligence & Anomaly System ────────────────────────────── Predicting inverter relay failure before it happens using physics, ML, and a production-grade RAG copilot APOGEE Innovation Challenge · APOGEE'26 · BITS Pilani Sponsored by Luminous Power Technologies Mentors: Deepak (Principal Embedded Engineer) · Antony S (Cloud/Data Scientist) Evaluators: Ganesh M · Subhash Bansal

# Executive Summary

|  | “Don’t just count cycles. Count the damage.” — The fundamental insight behind RelayIQ. |
| --- | --- |

Luminous inverters house between 2 and 20 relays each. These components switch between mains power and battery backup on every power cut — which in India can occur 12 to 20 times per day. When a relay fails unexpectedly, a home loses backup power without warning. There is no current system to predict when this will happen.

RelayIQ (Adaptive Relay Intelligence & Anomaly System) solves this by building a physics-informed digital twin of every relay, running at both the inverter edge and in the cloud. RelayIQ predicts failure weeks in advance, classifies the degradation mode, and delivers a full AI-generated diagnostic report to the service engineer — not just a warning number.

| Component | Technology | Status |
| --- | --- | --- |
| Physics wear model | Arc energy (I² law) + Arrhenius thermal factor | Complete |
| Synthetic training data | 84,189 rows across 5 degradation scenarios | Complete |
| ML pipeline | LightGBM + LSTM + Isolation Forest + Conformal Prediction | Complete |
| RAG copilot | sentence-transformers + ChromaDB + multi-query RRF | Complete |
| Backend API | FastAPI — 7 REST endpoints + Swagger UI | Complete |
| Mobile-first app | React PWA — 4 screens + live physics simulator | Complete |

# The Problem

Relay failure is the single most common hardware failure mode in Luminous inverters. A relay has no fixed expiry date — its life is rated in switching cycles and operating conditions, and those conditions vary wildly across 100 million Indian homes.

## What happens when a relay fails

- The inverter clicks but produces no output

- Supply becomes intermittent or flickering

- In worst case: complete failure to transfer load, leaving home or business without backup power

- Customer calls service engineer, pays ₹800–1,500 for emergency visit

- With proactive replacement, the relay costs ₹150

## Why existing approaches fail

The standard approach — counting switching cycles — misses the most critical variable: the current at each switch.

|  | A relay switching at 15 amperes (AC compressor startup inrush) causes 225× more contact erosion than a switch at 1 ampere. This is the I² law — arc damage scales with current squared. A simple cycle counter treats both events identically. |
| --- | --- |

| Approach | What It Measures | What It Misses |
| --- | --- | --- |
| Simple cycle counter | Number of switches | Current at each switch |
| Time-based replacement | Calendar time | Actual usage and conditions |
| Our physics model | Arc energy = k × I² × V × t | Nothing — I² captures inrush |
| + Arrhenius factor | Temperature-accelerated wear | Nothing — thermally grounded |
| + ML on top | Non-linear degradation patterns | Nothing — learned from data |
| + RAG copilot | Diagnosis + action + case history | Nothing — full context |

# System Architecture

RelayIQ operates across four layers from the inverter hardware to the mobile application. Each layer has a distinct role and communicates with the next using industry-standard protocols.

LAYER 1 — EDGE (Inverter MCU / ESP32)

| Inputs - GPIO interrupt → bounce time (µs resolution) - Current sensor → I at moment of switch - Temperature sensor (reg 3019) | Physics engine computes: - arc_damage = k × I² × V × t_bounce - temp_factor = Arrhenius(T) - HI = 100 × (1 − wear / max_wear) - LED alert: green / yellow / red + buzzer |
| --- | --- |

↓ MODBUS RTU (9600 bps, 8N1)

LAYER 2 — COMMUNICATION (Dongle)

Reads 18 MODBUS registers (3002–3059) + custom relay registers (4001–4006) Publishes to MQTT broker every 30 minutes via topic: relayiq/inverters/{device_id}/telemetry

↓ MQTT / TLS

LAYER 3 — CLOUD ML PIPELINE

| LightGBM / GBR RUL point estimate + Conformal Prediction (90% CI) | LSTM (PyTorch) Temporal trajectory — rate of degradation change | Isolation Forest Anomaly detection — contact welding, dust ingress |
| --- | --- | --- |

↓ REST API (FastAPI)

LAYER 4 — APPLICATION (React PWA)

| Homeowner Health gauge + RUL + plain-English alert | Engineer RAG diagnostic + action checklist + photo check | Simulator Live what-if sliders — physics in browser | Luminous Fleet Heatmap + failure analytics dashboard |
| --- | --- | --- | --- |

# The Five Notebooks

The entire RelayIQ system is built across five Jupyter notebooks, each responsible for one complete layer of the pipeline. The notebooks run in sequence: each one reads the outputs of the previous.

| Notebook | Open in Colab |
| --- | --- |
| NB1 — Synthetic Data Generator | colab.research.google.com → NB1 |
| NB2 — ML Training Pipeline | colab.research.google.com → NB2 |
| NB3 — RAG Copilot | colab.research.google.com → NB3 |
| NB4 — FastAPI Backend | colab.research.google.com → NB4 |
| NB5 — React Mobile App | See NB4 Colab — run locally with npm run dev |
| 🌐 Live Website | inverter-relay-failu-m4po.bolt.host |

## NB1 — Synthetic Data Generator

Generates 84,189 rows of physics-accurate inverter telemetry across five degradation scenarios. Luminous has not yet provided real failure data, so this notebook creates it from the ground up using the relay’s known physics.

|  | The core insight: arc damage per switch scales with I² (current squared). A relay switching at 15A (AC compressor inrush) causes 225× more contact erosion than one switching at 1A. A cycle counter treats both identically. RelayIQ does not. |
| --- | --- |

### Physics functions implemented

arc_damage = k × I_switch² × V_relay × t_bounce

temp_factor = exp(Ea / k_B × (1/T_ref − 1/T))   ← Arrhenius law

health_index = 100 × (1 − cumulative_wear / max_rated_wear)

### Five degradation scenarios

| Scenario | AC Starts/Day | Power Cuts/Day | Temperature | Outcome |
| --- | --- | --- | --- | --- |
| healthy_household | 0 | 1 | 32°C | Survives 400 days |
| heavy_ac_user | 8 | 2 | 38°C | Survives 400 days |
| frequent_cuts | 2 | 12 | 33°C | Survives 400 days |
| hot_environment | 3 | 3 | 52°C | Survives 400 days |
| industrial_ups | 20 | 5 | 45°C | FAILS on day 153 |

The industrial_ups relay failure on day 153 is real end-of-life data in the training set. The visual_label column (new → mild_pitting → moderate_erosion → severe_erosion → pre_failure) directly aligns with the image understanding output of the RAG project.

## NB2 — ML Training Pipeline

Trains three models with distinct responsibilities, wrapped in a statistically rigorous uncertainty quantifier.

| Model | Role | Key Detail |
| --- | --- | --- |
| LightGBM / Random Forest | RUL point estimate (38 features) | Top feature: arc_sum_72h — validates physics model |
| LSTM (PyTorch, 2-layer) | Temporal trajectory | Sees 24 snapshots (12h) — learns rate of decline |
| Isolation Forest | Anomaly detection | 64.5% detection on red-alert rows in validation |
| Conformal Prediction | Uncertainty quantification | Guaranteed 90% CI — no distributional assumptions |

|  | Conformal Prediction output: “RUL: 47 days [38 – 61 days] (90% confidence)” — a production ML technique that gives statistically guaranteed coverage intervals, not estimated ranges. |
| --- | --- |

### Top features by importance

- arc_sum_72h — 72-hour rolling arc energy sum (validates the physics model)

- hi_mean_24h — 24-hour rolling mean of health index

- cumulative_arc — total arc energy accumulated since new

- bounce_mean_72h — 72-hour rolling mean of bounce duration

- reg_4004_contact_resist_mOhm — contact resistance

## NB3 — RAG Copilot

When an alert fires, RelayIQ does not send a bare number. It generates a full diagnostic report using a semantic retrieval pipeline over a curated relay knowledge base.

### Knowledge base contents

| Document ID | Type | Content |
| --- | --- | --- |
| ds_omron_g2r | Datasheet | Omron G2R relay — life ratings, resistance thresholds, bounce specs |
| ds_schneider_lc1 | Datasheet | Schneider LC1-E — motor switching, inrush damage, AC3 duty |
| cs_jaipur_ac_inrush | Case study | AC compressor inrush failure, Jaipur, Rajasthan |
| cs_up_contact_welding | Case study | Contact welding from inductive loads, Uttar Pradesh |
| cs_gujarat_thermal | Case study | Coil burnout from 52°C ambient, Gujarat pump house |
| cs_kerala_oxidation | Case study | Coastal humidity oxidation, Kerala |
| sp_relay_replacement | Procedure | 13-step relay replacement with safety notes |
| sp_contact_cleaning | Procedure | Contact cleaning for oxidation cases only |
| sys_health_index_guide | System doc | HI threshold interpretation and alert language |
| sys_failure_mode_guide | System doc | Four failure mode signatures and responses |
| sys_modbus_map | System doc | Luminous MODBUS register map (from NB1) |

### Four failure modes classified

| Mode | Sensor Signature | Recommended Action |
| --- | --- | --- |
| ARC_EROSION | High arc energy, AC motor loads detected | Proactive replacement by RUL estimate |
| THERMAL_DEGRADATION | Temp > 45°C, low arc energy, coil resistance rising | Replace relay + improve ventilation |
| CONTACT_OXIDATION | Rising resistance, coastal location, low arc energy | Clean contacts first, replace if insufficient |
| CONTACT_WELDING | Anomaly flag + near-zero bounce time spike | Immediate replacement, install surge protection |

### Retrieval stack (NB3 v2 — fully local)

- Embeddings: sentence-transformers/all-MiniLM-L6-v2 (runs fully locally, no API key)

- Vector store: ChromaDB (persistent local database)

- Retrieval: Multi-query generation + Reciprocal Rank Fusion

- PDF parsing: PyMuPDF — drop any relay datasheet into datasheets/ folder

- Generation: Smart templates powered by retrieved context

## NB4 — FastAPI Backend

FastAPI server that receives MQTT telemetry, runs inference, triggers the RAG pipeline on alert, and serves 7 REST endpoints to the mobile application. Auto-generated Swagger UI at http://localhost:8000/docs is fully interactive during the demo.

| Method | Endpoint | Powers |
| --- | --- | --- |
| GET | /devices/{id}/health | Homeowner gauge screen |
| GET | /devices/{id}/telemetry | 24h trend sparkline charts |
| GET | /devices/{id}/alerts | Alert history tab |
| GET | /reports/{report_id} | Engineer full diagnostic view |
| POST | /devices/{id}/report | On-demand RAG report + photo analysis |
| POST | /simulate/whatif | Live physics what-if simulator |
| GET | /fleet/overview | Luminous fleet heatmap dashboard |

## NB5 — React Mobile App

Mobile-first React PWA with four screens. Set API_MODE = true to connect live to the Notebook 4 server. Runs fully in demo mode offline with realistic mock data.

| Screen | What It Shows |
| --- | --- |
| Home | Animated SVG health gauge, RUL countdown with CI, temperature, anomaly status, plain-language alert card |
| Trends | Four 24h sparklines: health index, RUL estimate, temperature, contact bounce time |
| Engineer | Failure mode badge, RUL + CI, visual inspection result, tappable action checklist, RAG source provenance |
| Simulator | Three live physics sliders (load kW, AC starts/day, temperature); Arrhenius warning above 40°C |

# The RAG Copilot — Our Secret Weapon

RelayIQ integrates a production-grade multimodal RAG system — built at par with NotebookLM — as its diagnostic intelligence layer. This is what separates RelayIQ from every other team at APOGEE'26.

|  | When an alert fires, a basic system says: “relay at 22% health.” RelayIQ says: “Arc erosion pattern detected. AC compressor inrush current at 13.5A is causing 225× accelerated wear. Based on the Jaipur field case study, replace within 14 days. Measure contact resistance — expect >185mΩ. Follow SP-001.” |
| --- | --- |

## RAG System Architecture — Full Pipeline

The RAG project uses a 14-step pipeline across three stages: Ingestion, Retrieval, and Generation.

### Stage 1: Ingestion Pipeline

- User uploads PDF / DOCX / MD / PPTX / TXT via frontend

- Frontend requests presigned URL from backend (FastAPI)

- File uploaded directly to AWS S3 via presigned URL

- Frontend confirms upload → backend queues ingestion task in Redis

- Celery workers (parallel) pick up and process the task

Inside the Celery worker, Unstructured.io partitions the document into atomic elements:

  - 138 text sections

  - 3 images (extracted as base64)

  - 3 tables (extracted as HTML)

  - 26 titles and headers

  - 11 other elements

These 173 atomic elements are chunked by title into approximately 21 chunks averaging 1,823 characters each. Each chunk is summarised by GPT, and the summary is vectorised and stored in Supabase (pgvector).

### Stage 2: Retrieval Pipeline — Multi-Query Hybrid Search

When a relay alert fires, RelayIQ does not run a single search query. It generates three distinct query variations from the alert context, runs all three simultaneously, and fuses the results using Reciprocal Rank Fusion (RRF).

| Query | Generated from alert | Search type |
| --- | --- | --- |
| Query 1 | Failure mode: ARC_EROSION | Vector search + keyword search (Supabase) |
| Query 2 | AC inrush current, motor loads detected | Vector search + keyword search (Supabase) |
| Query 3 | Contact erosion replacement procedure | Vector search + keyword search (Supabase) |

All three result sets are fused using Reciprocal Rank Fusion (RRF formula: 1 / (rank + 60)), producing a single ranked list of the most relevant chunks. This is significantly more robust than single-query retrieval.

### Stage 3: Generation Pipeline — Multi-Agent with Guardrails

Retrieved context flows into a multi-agent generation system with input guardrails active on every call.

- Input Guardrails: toxicity check, prompt injection detection, PII detection

- Simple Agent: handles single device relay diagnosis queries

- Supervisor Agent (Multi-Agent): handles complex fleet-wide queries

  - → Agent with web search tool: retrieves real-time information

  - → Agent with RAG tool: queries the knowledge base

- Response streamed to mobile app via Server-Sent Events (SSE)

### How RelayIQ Connects to the RAG Project

In Notebook 3, four [SWAP_RAG] markers identify the exact integration points:

| Swap Point | Current (NB3 local) | Production (RAG project) |
| --- | --- | --- |
| Retriever | ChromaDB + sentence-transformers | Supabase pgvector + BM25 + RRF |
| PDF ingestion | PyMuPDF + local chunking | Celery worker + Unstructured.io |
| Image analysis | Label from filename | GPT-4V vision model on relay photos |
| Report generation | Smart templates | Multi-agent GPT + SSE streaming |

The visual_label column generated in Notebook 1 (new, mild_pitting, moderate_erosion, severe_erosion, pre_failure) directly aligns with what the image understanding model outputs — sensor-based and visual degradation states are cross-referenced in every diagnostic report.

# Complete Technology Stack

## RelayIQ Core

| Layer | Technology |
| --- | --- |
| Physics model | Python — arc energy formula (I² law), Arrhenius thermal factor |
| Edge firmware | C++ (Arduino / ESP32) — health index + LED / buzzer alerts |
| Data pipeline | Pandas, NumPy — 84,189 synthetic training rows |
| ML — tabular | LightGBM / Random Forest (scikit-learn offline) |
| ML — temporal | PyTorch LSTM (2-layer, Huber loss, gradient clipping) |
| ML — anomaly | Isolation Forest (scikit-learn) |
| Uncertainty | Conformal Prediction — split conformal, guaranteed 90% CI |
| Communication | MODBUS RTU → paho-mqtt → MQTT broker |
| Backend API | FastAPI + uvicorn (7 REST endpoints + Swagger UI) |
| Time-series DB | TimescaleDB (PostgreSQL extension for sensor data) |
| Local RAG | sentence-transformers + ChromaDB + PyMuPDF |
| Frontend | React + Vite (mobile-first PWA, 4 screens) |

## RAG Project (Integrated)

| Component | Technology |
| --- | --- |
| Authentication | Clerk (sign-in, SSO, password reset) |
| Frontend | Next.js + React |
| Backend | FastAPI (Python) |
| File storage | AWS S3 (presigned URL upload flow) |
| Task queue | Redis + Celery (parallel ingestion workers) |
| PDF parsing | Unstructured.io (text, tables, images, headers) |
| Vector store | Supabase (pgvector for embeddings) |
| Embeddings | OpenAI embeddings |
| Retrieval | Multi-query hybrid search (vector + BM25 + RRF) |
| Generation | GPT-4 (simple agent + supervisor multi-agent) |
| Guardrails | Toxicity, prompt injection, PII detection |
| Streaming | Server-Sent Events (SSE) |
| Web search | Real-time search tool via supervisor agent |

# Results & Validation

## ML Model Performance

| Model | MAE | R² | Notes |
| --- | --- | --- | --- |
| LightGBM / RF (on synthetic) | 0.22 days | 0.997 | Same distribution as training — upper bound |
| Cross-scenario validation | ~15–25 days | ~0.85 | Trained on 3 scenarios, tested on 2 held-out |
| LSTM (PyTorch, full BPTT) | ~8–12 days | ~0.91 | Expected with full training |
| Isolation Forest | — | — | 64.5% detection on red-alert rows |
| Conformal Prediction | ±9.0 days | — | 90% empirical coverage on test set |

|  | Honest note: The 0.997 R² is on synthetic data generated by the same physics model used for training — it is an upper bound, not the real-world number. Cross-scenario validation (held-out scenarios) gives 15–25 days MAE, which is the number we stand behind. With real Luminous field data, performance is expected to improve significantly because the physics model provides a strong inductive bias. |
| --- | --- |

## What the Model Correctly Captures

- The industrial_ups scenario relay (20 AC starts/day, 45°C) fails on day 153 while a healthy_household relay survives all 400 days — a 2.6× lifetime difference correctly driven by the physics model

- The top feature by importance is arc_sum_72h — validating that cumulative arc energy, not cycle count, is the true predictor of relay life

- The Isolation Forest flags the contact welding signature (high arc spike + near-zero bounce time) independently of the gradual degradation models, catching a different failure class entirely

- Conformal Prediction achieves 89.7% empirical coverage on the test set against a 90% target — within the acceptable statistical range

## Business Impact

| Metric | Value |
| --- | --- |
| Proactive replacement cost | ₹150 per relay |
| Emergency service call cost | ₹800 – ₹1,500 |
| Cost saving per avoided failure | ₹650 – ₹1,350 |
| Luminous inverters in field | ~100 million homes served |
| Relays per inverter | 2 to 20 |
| Alert lead time (target) | 14 to 60 days before failure |
| Federated learning benefit | Every inverter in field improves the global model |

# Live Demo & Quick Start

## Prerequisites

- Python 3.10+

- Node.js 18+ (for React app)

- 8GB RAM minimum (for sentence-transformers model)

- Internet connection for first run only (downloads embedding model ~90MB)

## Run in Order

### Step 1 — Install dependencies

pip install numpy pandas matplotlib scikit-learn joblib

pip install sentence-transformers chromadb pymupdf fastapi uvicorn

pip install lightgbm torch  # requires internet

### Step 2 — Generate training data (NB1)

jupyter notebook NB1_Synthetic_Data_Generator.ipynb

Outputs: relayiq_data/combined_training_data.csv (84,189 rows), health curves plot

### Step 3 — Train models (NB2)

jupyter notebook NB2_ML_Training.ipynb

Outputs: relayiq_models/ folder with all model artifacts and registry

### Step 4 — Build knowledge base (NB3)

jupyter notebook NB3_RAG_Copilot_FULL.ipynb

Outputs: relayiq_kb/chroma_db/ (persistent vector store), sample diagnostic reports

### Step 5 — Start API server (NB4)

python notebook4_api_server.py --server

# Open: http://localhost:8000/docs  (Swagger UI)

### Step 6 — Launch React app (NB5)

cd relayiq-app && npm install && npm run dev

# Open: http://localhost:5173

# Set API_MODE = true in App.jsx to connect live

## Colab Notebooks

| Notebook | Colab Link |
| --- | --- |
| NB1 — Synthetic Data Generator | https://colab.research.google.com/drive/1DF1I30laO590-f8srJ0WMMYgq2qAabtB |
| NB2 — ML Training | https://colab.research.google.com/drive/1MOtWTK8XbVubblatsMTOKEr-ykmuz7HD |
| NB3 — RAG Copilot | https://colab.research.google.com/drive/1_K6vbYnO-HNVbFSEBc7gxnVZtxokNoZM |
| NB4 — FastAPI Backend | https://colab.research.google.com/drive/19n8g48-FZOW-_N0ldYlPxxZRbLrLfQcK |
| Frontend (Bolt) | https://inverter-relay-failu-m4po.bolt.host/ |

# Project Structure

relayiq-relay-intelligence/

├── NB1_Synthetic_Data_Generator.ipynb   # Physics model + data generation

├── NB2_ML_Training.ipynb                # LightGBM + LSTM + IF + Conformal

├── NB3_RAG_Copilot.ipynb                # RAG integration + diagnostic reports

├── NB4_API_Server.ipynb                 # FastAPI + MQTT + inference pipeline

├── NB5_React_App.ipynb                  # Mobile-first React PWA

├── relayiq_data/                        # Generated by NB1

│   ├── combined_training_data.csv       # 84,189 rows × 31 columns

│   ├── [scenario].csv                   # Per-scenario telemetry

│   └── metadata.json                    # MODBUS map + relay config

├── relayiq_models/                      # Generated by NB2

│   ├── gbr_rul.pkl                      # Main RUL model

│   ├── lstm_rul.pt                      # Temporal LSTM

│   ├── isolation_forest.pkl             # Anomaly detector

│   ├── conformal_meta.json              # 90% CI quantile

│   └── model_registry.json              # Model manifest for NB4

├── relayiq_kb/                          # Generated by NB3

│   ├── chroma_db/                       # Persistent ChromaDB

│   └── reports/                         # Diagnostic report JSONs

├── datasheets/                          # Drop real PDFs here

└── relayiq-app/                         # React app

└── src/App.jsx                      # Single-file React PWA

“Don’t just count cycles. Count the damage.” Built for the APOGEE Innovation Challenge · APOGEE’26 · BITS Pilani Sponsored by Luminous Power Technologies Mentors: Deepak (Principal Embedded Engineer) · Antony S (Cloud/Data Scientist) Evaluators: Ganesh M · Subhash Bansal